Innovative Technology Notice

Application Submitted Pursuant to Title 5, 310 CMR 15.000

Department of Environmental Protection

Watershed Permitting Program

#### Transmittal Number:  W055433

# Application Filing Date: October 22, 2004

Public Notice is hereby given that the Watershed Permitting Program of the Department of Environmental Protection is proposing Certification for General Use for a Presby Enviro-Septic System pursuant to 310 CMR 15.288 of Title 5. This provision of Title 5 describes the approval process for General Use of innovative/ alternative technologies for the treatment and subsurface disposal of sewage from facilities regulated under Title 5.

The Watershed Permitting Program proposes the use of alternative standards for the following provisions in Title 5:

- That the setback distance as specified in 310 CMR 15.211 (1) [4] is not applicable to the Presby Enviro-Septic System.

- That the use of aggregate for soil absorption systems, as specified in 310 CMR 15.247, is not applicable to the Presby Enviro-Septic System.

- That the Presby Enviro-Septic System consisting of a 11 5/8-inch diameter corrugated, high-density plastic pipe, perforated with holes equally distributed around its inner circumference and with the exterior of the pipe covered with two layers of fabric, an inner layer of coarse, randomly oriented polypropylene fabric and an outer layer of non-woven geo-textile polypropylene fabric, and installed in a concrete sand bed surrounded on all sides by a minimum of six inches of sand, is equivalent to gravel and distribution pipe systems when the area required for effluent disposal in accordance with 310 CMR 15.242 and 15.252 is reduced by 50%.

- That the bed design specifications, as specified in 310 CMR 15.252 (2), are not applicable to the Presby Enviro-Septic System.

- That the distribution box, as specified in 310 CMR 15.232 (1) is not required where a Serial layout of the Presby Enviro-Septic System is installed.

The Department welcomes any interested parties to provide any relevant information concerning the performance of this technology to be considered in the review process.  The Department will consider all data and written comments submitted within 21 days after the publication of this notice in order to decide whether to grant approval pursuant to 310 CMR 15.000.  To schedule a time to review the research and other data available or to obtain further information regarding this proposal please contact the Watershed Permitting Program at (617) 292-5920.  All written comments should be submitted to: Director, Department of Environmental Protection, Watershed Permitting Program, One Winter Street, 6th Floor, Boston, MA 02108.

If you wish notification of the Department's preliminary determination concerning this proposal, please send to the Department a written request with your comments, if any, within 21 days from the publication date of this Environmental Monitor.